Draft basin plan priority assets classification – Victorian wetlands

Description of wetland attributes

Climate zone

Valid options: Semi-arid, Temperate inland, Temperate upland, Temperate alpine.

Description: The climate attribute describes the Koppen climate class that the wetland falls within.

Data source: Kőppen climate map spatial layer from BOM website.

Method: Using the shapefile of the Kőppen climate zones overlayed with the Wetlands 1994 layer the climate zone could be assigned accurately. Victoria has two Kőppen climate zones: Grassland and Temperate. The climate zones were modified to better reflect the climatic regions of Victoria using the methodology of the from the NSW MER program (reference?). The climate zones in NSW are similar to Victoria so it was deemed suitable to apply the same naming convention. The alterations were to rename grassland to ‘semi-arid’ and temperate which is split into alpine, upland and inland.

The climatic classes used for Victorian typology are:

- Semi-arid

- Temperate

  - Temperate Inland

  - Temperate Upland (700 – 1800m)

  - Temperate Alpine (>1800m)

The majority of sites for fell within the Semi-arid or Temperate inland zone.

Limitations: The Kőppen climate mapping is very broad.

Water source

Valid options: River, Local, Groundwater.

Description: The origin of the primary water source for the wetland system.

Data source: Hydrology layer and wetlands mapping.

Method: River includes wetlands that are filled directly from a river. Local means the wetland is predominantly filled through rainfall in the local area. Groundwater fed wetlands are filled from groundwater that is close to the surface.

Limitations: Many wetlands have multiple water sources and it is difficult to ascertain the relative contribution of these water sources. Irrigation system fed wetlands are difficult to assess.

Water regime

Valid options: Frequency – Permanent, Seasonal, Ephemeral. Duration – Permanent, <=1 year, >1 year.

Description: The frequency of water flowing into the wetland. Permanent means that water is constantly flowing in. Seasonal means water is flowing into the wetland at least, or more than, once a year. Ephemeral means that water flows into the wetland less than once a year.

The duration of the water remaining in the wetland. Permanent means that water remains in the wetland throughout the year. <=1 year means that water remains in the wetland for less than or equal to one year. >1 year means that water remains in the wetland for a period greater than one year but is not permanent.

Data source: Corrick & Norman (1980) wetland classification definition (see table below).

Method: Water regime frequency and duration options were allocated for each of the six Corrick & Norman (1980) wetland classifications based on the period of inundation and depth information from the wetland mapping.

| Corrick classification | Period of inundation | Frequency | Duration |
| --- | --- | --- | --- |
| Freshwater Meadow (FM) | <4 months/year | Seasonal | <=1 year |
| Shallow Freshwater Marsh (SFM) | <8 months/year | Seasonal | <=1 year |
| Deep Freshwater Marsh (DFM) | >8 months/year - permanent | Permanent | <=1 year |
| Permanent Open Freshwater (POF) | Permanent | Permanent | Permanent (some >1 year) |
| Semi-Permanent Saline Wetland (SPSW) | <12 months/year | Seasonal | <=1 year |
| Permanent Saline Wetland (PSW) | Permanent | Permanent | Permanent |

Limitations: Many ‘permanent’ wetlands have become dry in the past 5 years so the classification is not accurate during drought conditions. Potential errors with the Wetland 1994 layer.

Water type

Valid options: Fresh, Saline

Description: Salt concentration in the water of the wetland.

Data source: Corrick & Norman (1980) wetland classifications. Wetland 1994 layer.

Method: Assign option from the wetland type (see table above). Two out of the six wetland types are saline, the remaining four are fresh.

Limitations: Potential errors with the wetland mapping.

Soil/substrate

Valid options: Rock, Sand, Mineral, Organic.

Description: This attribute was deemed to difficult to assign and was not completed.

Vegetation

Valid options: Tree, Shrub, Grass/herb/sedge, Submerged.

Description: Presence of water dependent native vegetation at the wetland.

Data source: Corrick & Norman (1980) wetland classification sub-category and Heron & Joyce (2008) for some wetlands.

Also DSE (2004) for Kerang wetlands.

Method: The dominant vegetation type was listed from the information available.

Limitations: Would be good to use aerial photography to confirm the vegetation groupings. Wetland vegetation mapping is not available.

Landform

Valid options: Basin, Flat.

Description: The landform attribute is used to describe the landform of the wetland itself and not the land surrounding it.

Data source: Hydrology 1:100,000 layer. Jones & Miles (2009) used DEM for the S.A. River Murray Wetland Classification Project. DEM information isn’t readily available for all the wetlands listed for Victoria

Method: It is expected that all wetlands form basins for the listed sites in Victoria.

Limitations: DEM are not available for all wetlands.

Landscape context

Valid options: Floodplain, Non-floodplain

Description: Landscape context refers to the land surrounding the wetland and can be defined by alluvial plains characterised by frequently active erosion and aggradation by channelled or overbank stream flow (McDonald et al 1998).

The Non-floodplain category refers to systems that may receive some flow from local watershed creeks but are only very rarely or very minimally influenced by true river systems.

Data source: 1 in 100 year flood and Hydrology layers.

Method: Most sites were mapped as floodplain wetlands and were assigned as such if close to a major river system. If a wetland was mapped as a floodplain wetland but had a local water source it was assigned non-floodplain instead.

Limitations: The attribute is very broad but appropriate for a landscape scale classification.

Hydro-connectivity

Valid options: Terminal branch, Through-flow, Over-bank

Description: Hydro-connectivity refers to the hydrological connectedness of wetlands to other aquatic ecosystems.

Data source: Hydrology 1:100,000 layer, topographic mapping

Method: The shape of the hydrology polygon is used to distinguish the hydro-connectivity of wetlands to river systems.

Limitations: The hydrology polygon may not fully represent the hydrology of the wetland, so multiple sources of information need to taken into consideration. It is sometimes difficult to follow the hydrology connections at some wetlands, especially those that are part of an irrigation system (i.e. Torrumbarry Irrigation system).

References:

Corrick, A.H. and Norman, F. I. (1980) Wetlands of Victoria I. Wetlands and waterbirds of the Snowy River and Gippsland Lakes catchment, Proceedings of the Royal Society of Victoria. 91: 1-15

DSE (2004) Kerang wetlands Ramsar site strategic management plan, Department of Sustainability and Environment, East Melbourne

Heron, S. and Joyce, A. (2008) NRSWS Impact of water availability on significant wetlands. Report prepared for the DSE.

Jones, L. and Miles. M., 2009, River Murray Wetland Classification Project (DEH) report to the Riverine Recovery

Project, Department of Water, Land & Biodiversity Conservation.

McDonald, R.C., Isbell, R.F., Speight, J.G., Walker, J. and Hopkins, M.S. 1998, Australian Soil and Land Survey Field Handbook, Second Edition, CSIRO Land and Water, Canberra, Australia.

Quick reference table for the attributes

| MDBA classification Attributes | Categories | Source of Information |
| --- | --- | --- |
| Climate Zone | (Kőppen classification) Modified from NSW MER typology Grassland = semi-arid, Temperate = Temperate inland, Temperate upland, Alpine | Koppen Classification layer from BOM |
| Water Source | (Primary - river, local, groundwater) (link to NWC) | Hydrology (Hydro 1:100,000) or Heron & Joyce (2008) |
| Water Regime | (Frequency - Permanent, Seasonal, Ephemeral) and (Duration - Permanent, <= 1 year, > 1 year) | Corrick & Norman (1980) |
| Water Type | (Fresh, Saline) | Corrick & Norman (1980) |
| Soil/Substrate | (Rock, sand, mineral, organic) | - |
| Vegetation | (Tree, Shrub, Grass/herb/sedge, Submerged) | Corrick & Norman (1980), Heron & Joyce (2008) |
| Landform | (Basin, Flat) | Hydrology (Hydro 1:100,000) |
| Landscape context | (Floodplain, non-floodplain) | 1 in 100 year flood (EXTENT_100Y_ARI) |
| Hydro-connectivity | (Terminal Branch, Through-flow, Over-bank) | Hydrology (Hydro 1:100,000), Topographic mapping |
| Biodiversity data? – not using |  | - |

Note: the data layers Hydrology, 1 in 100 year flood, Wetlands 1994 are available to view on the Biodiversity Interactive Map.

www.dse.vic.gov.au/

Online services>Interactive Maps>Biodiversity Interactive Map